### Compressor Timing Models

To simulate a compressor station stochastically, one must develop a time series of the operating states, with transitions between them.  In rare cases, this may be acquired directly from SCADA records. In the more common case when these data are not available, the primary activity data driving emissions is an estimate of the run time of the compressor over the simulation period. This is often expressed as an uptime ratio, i.e. the fraction of time the compressor is operating.

For transmission, storage and gas processing stations, yearly averages can be extracted from GHGRP reports because operators report the number of hours per year in each operating mode, for each compressor unit.  For other sectors, annual data is not available, but often uptime can be estimated from SCADA records or other sources.

Uptime is defined as:

u=tOPtOP+tNOP+tNOD

where t* is the time in each mode. Often, only toff=tNOP+tNOD is known, and some estimate must be made for the fraction of time a unit spends in NOP versus NOD mode. To model runtimes, we use two steps: probability distributions for the time in a mode (states) and a transition matrix to define the probability for transition between modes (arrows). When a compressor enters a mode, the time in that mode is randomly drawn from an appropriate distribution.  (The example below uses triangular distributions, but others are possible.) When the time in-mode expires, the model uses the transition matrix to move to the next step.

Duration probability distributions: Each tx in the prior equation is assigned a probability distribution, Px, based upon analysis of other data or engineering judgment.  Mean values of these distributions must follow the prior equation to produce the correct uptime, see the appendix for more details on the calculations required to assure the probability distributions produce the correct mean uptime during simulation.

Transition matrix defines the probability of transitioning from one state to another whenever there is a choice in possible destination states. An example table is shown below:

| Current State | Future State |  |  |
| --- | --- | --- | --- |
|  | OP | NOP | NOD |
| OP | 0 | ΠOP,NOP | ΠOP,NOD |
| NOP | ΠNOP,OP | 0 | ΠNOP,NOD |
| NOD | ΠNOD,OP=1 | 0 | 0 |
| Note: Episodic states (blowdown and gas start) are not shown in the matrix, as they are completely dependent upon the mode transitions shown. |  |  |  |

Probabilities in each row sum to 1. Diagonal values are zero, because we are drawing duration from duration probability distributions to determine how long we are in each state. We also assume that transitions from NOD to NOP are very rare (Π≅0), or short – if a depressurized compressor is repressurized, it will also be started.

#### Calculation of Uptime Fractions

Computing length of time in off condition to meet the uptime requirement.  We are given the uptime fraction, u, and want to find the scale factor, f, that will give us that uptime factor, where a is the uptime in some convenient time unit, a toff=ftOP.  u is defined as:

u=μOPμOP+fμOP=11+f

f=1u-1

Since there are multiple downtime modes, we need to scale downtimes to considering the means and the fraction in each mode.  Assuming that NOD mode is more common the NOP, we start by estimating μNOD=fμOP. We then estimate μNOP using other information. With these estimates, we have an estimated up-time fraction:

uest=μOPμOP+μoff

where μoff=ΠNOPμNOP+ΠNODμNOD and Πx is the relative probability (i.e. ΠNOP+ΠNOD=1 where 0<Π*<1) of transitioning into either mode when leaving NOD mode. We desire uest=u by scaling both μNOP and μNOD by factor, g:

u=μOPμOP+gμoff

Solving for g:

g=μOPμoff1u-1

It may be desirable to also scale other parameters of the probability distributions when scaling μx.  Probabilities are extracted directly from the transition matrix to calculate ΠNOP and ΠNOD.

### Compressor Vents and Leaks

Compressors, like all pieces of major equipment, are assigned leaks in three steps:

- Assign components

- Determine which components are leaking at any given time based on the leak probability

- Assign emission factors to the leaking components

Each compressor is instantiated with a list of components. Each component has a probability of leaking, based on the quantity of that particular component, and the number of leaking components of that type identified in a field survey or reported to GHGRP. In some cases, this probability may be zero, where insufficient data exists.

All compressors are assumed to have the following components:

- One single unit vent

- One blowdown valve

- One starter valve

- Two isolation valves (one inlet and one outlet)

In addition, all reciprocating compressors are assumed to have one rod packing vent, and all centrifugal compressors have either wet or dry seal vents. Rod packing and wet/dry seal emissions are classified as vented emissions, rather than fugitive, but functionally behave as leaks and are treated as leaks by the simulation, although emissions are categorized as vented rather than fugitive.

Other components vary based on data source.

Compressor leaks differ from other leaks in that they may be state dependent. Table III lists all the possible leaks a compressor could have, and which state(s) the leaks will be active.

Table II. Compressor leak state table.

|  | RUNNING | NOP | NOD | STARTING | BLOWDOWN |
| --- | --- | --- | --- | --- | --- |
| Blowdown valve | X | X |  | X |  |
| Connector (Flanged) | X | X |  | X | X |
| Connector (Threaded) | X | X |  | X | X |
| Dry seal | X |  |  |  |  |
| Isolation valve |  | X | X |  | X |
| Meter | X | X |  | X | X |
| OEL | X | X |  | X | X |
| Pocket vent | X | X |  | X | X |
| PRV | X | X |  | X | X |
| Regulator | X | X |  | X | X |
| Rod Packing (OP) | X |  |  |  |  |
| Rod Packing (NOP) |  | X |  |  |  |
| Rod Packing (NOD) |  |  | X |  |  |
| Single unit vent | X | X |  | X |  |
| Starter valve | X | X | X |  | X |
| Wet seal | X |  |  |  |  |
| Valve | X | X |  | X | X |

### Compressor Power & Exhaust Modelling

#### Inputs

P 	- Engine rated power, in kW

l 	- Load on engine as a fraction of rated load, at any given time

ηe	- Engine efficiency as a ratio of shaft output (kW) to fuel input (kW … kJ/s)

N	- Number of gas species.

gs	- Fuel gas mass fraction for species s, as a mass ratio, kgkg

γs	- Fuel gas mass fraction for species s, in driver units, kg<driver>

Ns 	- Lower heating value (LHV) of gas species s. (REFPROP uses the term net heating value)

cs 	- Mass of CO2 emitted for complete combustion of gas species s during the combustion process, kgkg

B	- Driver value, in driver units of <driver>s, such as bbls or kW which is kJs

Note that

s=1Ngs=1

#### Compressor Power Model

The mass fuel flow rate through the prime mover is set by the power delivered by the prime mover.  Therefore, ms must be calculated from engine power.

Each time a prime mover starts, the load level may change.  Resulting total load on the engine is Pl (in kW).  Applying the efficiency of engine, the fuel power (heat rate) required is:

El=Plηe                kJs

While MEET currently uses a single efficiency value, in general efficiency is a function of load on the engine, relative to rated load, ϕe=PlPr where Pr is the rated load of the engine/driver.  In general, the function:

ηe=ηe(ϕe)

is implemented as a quadratic or cubic equation. El is the energy input rate – i.e. input fuel power – required by the engine/driver to produce Pl output power.  Internally in MEET, this is translated to kW, or more precisely, kJs.

The incoming fuel gas composition is defined in terms of kg<driver>; currently MEET always uses kg/scf. Based upon the driver, the heat rate of the incoming fuel is:

Nf=s=1NmsNs             kgscf∙kJkg=kJscf

where Ns is the heat rate – lower or higher heating value – of each species in kJkg.  Either higher or lower heating value can be used provided ηe is based on a consistent definition of the fuel’s heating value. Nf is the fuel’s heading value, in the fuel’s driver unit; for typical MEET fuel gas compositions, this would be in kJscf.

Dividing El provides the fuel flow rate to the engine or turbine:

Ql=ELNf         kJs∙scfkJ=scfs

For current implementation of MEET, Ql would be in units of scfs.  The incoming gas composition, fs, should be in kgscf, which allows the mass flow by species to be

ms=Qlfs=ElNffs=PlηeNffs=PlfsηeNf        kJs∙kgscf∙scfkJ=kgs

Assuming that the driver MEET outputs is the load on the engine, then by analogy to ms=γsB, the incoming, modified, gas composition for an engine is:

γs(e)=fsηeNf     [kgscf*scfkJ= kgkJ]

| Name | Carbon | Hydrogen | kgCO2perKgAlkane | kgH2OperKgAlkane | Lower heating Value, Ns (kJ/kg)* |
| --- | --- | --- | --- | --- | --- |
| METHANE | 1 | 4 | 2.743332215 | 2.245931009 | 50048 |
| ETHANE | 2 | 6 | 2.927235334 | 1.797367605 | 47800 |
| PROPANE | 3 | 8 | 2.994140808 | 1.634176531 | 46400 |
| BUTANE | 4 | 10 | 3.028753708 | 1.54975121 | 44862 |
| ISOBUTANE | 4 | 10 | 3.028753708 | 1.54975121 | 45300 |
| PENTANE | 5 | 12 | 3.049908257 | 1.498152537 | 45241 |
| ISOPENTANE | 5 | 12 | 3.049908257 | 1.498152537 | 45400 |
| HEXANE | 6 | 14 | 3.064176231 | 1.46335111 | 44752 |
| HEPTANE | 7 | 16 | 3.074449648 | 1.438292924 | 44925 |
| OCTANE | 8 | 18 | 3.082200032 | 1.419388738 | 44786 |

Table 1 Combustion Factors

*References

https://www.engineeringtoolbox.com/heating-values-fuel-gases-d_823.html

https://link.springer.com/content/pdf/bbm%3A978-1-4419-7943-8%2F1.pdf

Some companies do not leave stopped compressors in a pressurized state, while others use that mode extensively.  For example, storage compressors may be in NOP mode for extended period to provide fast response to gas demand changes.